Q. 1. Find the capacity and the rotational latency of the hard drive

Find the capacity and the rotational latency of the hard drive / How to calculate the hard disk capacity and its rotational latency?

Question:

1) Suppose that we have a hard drive with the following characteristics. It has 10,000 cylinders, 10 tracks per cylinder, 60 sectors per track, and 500 bytes per sector. The hard drive spins at 7200 rpm (rotations per minute) and has an average seek time of 10 ms. Which of the following combination is the right i) capacity and ii) rotational latency of the hard drive?

(a) 2 GB and 3.17 ms

(b) 3 GB and 4.17 ms

(c) 4 GB and 5.17 ms

(d) 1 GB and 2.17 ms

Answer:

b) 3 GB and 4.17 ms

Discussion:

Given,

Number of cylinders                      -        10,000

Number of tracks per cylinder      -        10

Number of sectors per track          -        60

Number of bytes per sector            -        500

Hard drive spinning speed           -        7200 rpm

i) Capacity of the hard drive

Capacity = cylinders * tracks per cylinder * sectors per track * bytes per sector

= 10000 * 10 * 60 * 500 = 3,000,000,000 bytes

= 3 GB

ii) Rotational latency of the hard drive

Time taken for one rotation = 1/RPM of the hard drive

= 1/7200 per minute

= (1/7200) * 60 = 0.0083 seconds

= 8.33 ms

Rotational latency = Half of the time consumed for one full rotation

= ½ * 8.3 ms = 4.17 ms

QUESTION:Find the capacity of a hard disk in database, How many recrods can be stored in a hard disk track? Which of the following is the correct combination of capacity of track, capacity of each surface, and capacity of disk?

Question:

Consider a disk with a sector size of 512 bytes, 14409 tracks per surface, 16947 sectors per track, 8 double-sided platters, and an average seek time of 9 ms. Which of the following is the correct combination of capacity of track, capacity of each surface, and capacity of disk (approximately)?

a) 100 MB, 200 GB, and 2 TB

b) 8 MB, 116 GB and 2 TB

c) 8 MB, 200 GB and 1 TB

d) 80 MB, 116 GB and 1 TB

Answer:

b) 8 MB, 116 GB and 2 TB

Discussion:

Given,

Sector size                                                  - 512 bytes

Number of sectors per track                    – 16947

Number of tracks per surface                  – 14409

Number of platters                                   – 8 double-sided

Answer:

Capacity of a track = sector size * number of sectors per track

= 512 * 16947 = 8 MB approx.

Capacity of each surface = capacity of track * number of tracks per surface

= (512*16947) * 14409 = 116 GB approx.

Capacity of the disk = capacity of each surface * number of platters * number  of pages per platter

= 116 GB * 8 * 2 = 2 TB approx.

3.

Question:

5) Consider a disk with the following specifications: 512 bytes sector size, 8 double sided platters (2 surfaces per platter), 5000 tracks per surface, 100 sectors per track, average seek time of 10 msec, and the disk rotates at 5400 revolutions per minute (rpm). Which of the following is the correct combination of number of cylinders, average rotational delay, and transfer rate (if an entire track of data can be transferred per revolution of disk)?

a) 5000 cylinders, 5.55 milliseconds, 4.5 KB

b) 5000 cylinders, 4.5 milliseconds, 12 KB

c) 4000 cylinders, 5.55 milliseconds, 10 KB

d) 4000 cylinders, 2 milliseconds, 2.5 KB

Answer:

a) 5000 cylinders, 5.55 milliseconds, 4.5 KB

Discussion:

Given,

Number of platters                -        8

Tracks per disk surface         -        5000

Sectors per track                    -        100

Sector size                               -        512 bytes

Average seek time                -        10 msec

rpm                                        -        5400.

Number of cylinders is same as the number of tracks. Hence, the number of cylinders is 5000.

The average rotational delay is half of the rotation time. One complete rotation takes 1/5400 in a minute = (1/5400) * 60 seconds = 0.0111 seconds = 11.1 milliseconds. The average rotational delay = 11.1/2 = 5.55 msec (approx).

We need the capacity of the track to find the transfer rate. The capacity of the track can be found as follows;

Number of bytes per track = bytes per sector * sectors per track

= 512 * 100 = 51200 bytes = 50 KB.

We know that, from above, the time for one complete rotation is 11.1 milliseconds. Hence, the transfer rate is 50 KB/11.1 milliseconds = 4.5 KB/msec (approx.).

Question:

2) Consider a hard disk with a sector size of 512 bytes, and 50 sectors per track. A block may be of several KBs in size. Which of the following is the valid block size?

(a) 256 bytes

(b) 786 bytes

(c) 4096 bytes

(d) 26112 bytes

Answer:

(C) 4096 bytes/4 KB

Discussion:

A block (cluster) is a unit of memory used to specify the cluster of one or more sectors (physical disk area). Block is the term used by operating systems to specify the minimal size of memory that can be accessible from hard disk.

Sector is the minimal size of physical storage (hard disk) and is the subdivision of a track of the hard disk.

The block size should be a multiple of sector size.

(a) 256 bytes – it is half of a sector. We need at least a sector. WRONG

(b) 786 bytes – it is not a multiple of sector size. WRONG

(c) 4096 bytes – it is a multiple of given sector size. 512 * 8 = 4096. RIGHT

(d) 26112 bytes – it is indeed a multiple of the sector size. 512 * 51 = 26112. But, it exceeds the size of a track (Given, 512 bytes * 50 tracks = 25600 bytes). The size of block (or cluster) cannot exceed the size of a track. Hence, WRONG.

Option (c), 4096 bytes is the valid block size.

3) Consider a disk with following characteristics; Block size 512 bytes, number of blocks per track 25 and number of tracks per surface 500. Also assume that there is a table EMPLOYEE with 10000 fixed length records and each record is of size 101 bytes. Which of the following is the correct bfr (blocking factor) and number of blocks combination if we assume an unspanned organization?

a) 5 and 2000

b) 5 and 1000

c) 6 and 2000

d) 6 and 1000

Answer:

a) 5 and 2000

Discussion:

Given,

Block size                                                  - 512 bytes (required)

Number of blocks per track                     – 25 (Not required)

Number of tracks per surface                  – 500 (Not required)

Number of records in EMPLOYEE        – 10000 (required)

Record size                                                         - 101 bytes (required)

Find bfr and number of blocks required to store entire EMPLOYEE records.

Spanned/Unspanned records – For example, if one block is of size 10 bytes and we have records of size 3 bytes each. Then each block can contain only 3 records. 3 * 3 = 9 bytes. We cannot store fourth record first byte in one block and the rest in the next block. This kind of records organization is unspanned. If we span the record in two blocks, then the organization or records is spanned records.

Blocking factor (bfr) – Number of records fit per block.

For spanned records, bfr = ceiling(B/R) where B is the block size and R is the record size.

For unspanned records, bfr = floor(B/R).

Answer:

Bfr for unspanned records = floor(B/R) = floor(512/101) = floor(5.07)

= 5

Number of blocks required = ceiling(r/bfr) where r is the number of records in the table.

= ceiling (10000/5) = 2000

Hence, bfr is 5 and the number of blocks is 2000.

QUESTION

Consider a disk with a sector size of 512 bytes, 1000 tracks per surface, 25 sectors per track, 5 double-sided platters, and average seek time of 10 msec. Choose the correct answer for the following;

(i) capacity of tracks in bytes,

(ii) number of cylinders the disk has,

(iii) the average rotational latency if disk rotates at 5400 rpm.

(a) 12.5 K, 1000, 5.5 msec

(b) 25 K, 2000, 11 msec

(c) 12.5 K, 1000, 11 msec

(d) 25 K, 1000, 100 msec

Answer:

(a) 12.5 K, 1000, 5.5 msec

Given,

Sector size (minimum storage unit of a hard drive) – 512 bytes

Number of double sided disk platters - 5

(One can read/write on both sides in a Double sided platters unlike DVD for example)

Number of tracks per each side of each disk platter – 1000

Number of sectors per track – 25

Average seek time – 10 msec

The time needed for moving head of a disk drive from one track to another is the seek time. The average of many seeks is average seek time.

(i) Capacity of track can be calculated as follows;

Capacity of track = bytes/track = bytes/sector * sectors/track

= 512 bytes/sector * 25 sectors/track = 12800 bytes

= 12800/1024 KB = 12.5 KB

(ii) The number of cylinders is the same as the number of tracks on each platter.

Number of cylinders = Number of tracks = 1000

(iii) Maximum rotational delay can be calculated as follows;

Maximum rotational delay = (1/rpm) * 60

= (1/5400) * 60 = 0.011 seconds or 11 msec

Average rotational latency is half of the rotation time, ie., 5.5 msec